The Study of an Optimal Ratio for Liquid Plaster with Antibacterial Activity Against Staphylococcus aureus Using Galangal Essential Oil Extract.

Natthabhorn A-runseangsil and Wareekan Kaowpong*

Princess Chulabhorn Science High School Phetchaburi , Kaoyai , Cha-Am , Phetchaburi

*E-mail : 64wareekarn.kha@pccphet.ac.th

Abstract

Acute wounds located in areas of frequent movement and in regions with limited

surface area for applying sheet-type plasters present challenges for protecting the wound

from environmental exposure. This is especially true for small wounds or those situated in

hard-to-reach areas such as periungual regions or finger joints, which are also at increased

risk of infection from skin microorganisms. This study aimed to develop a liquid bandage

containing galangal essential oil that can form a conformable, flexible wound-sealing film

with antimicrobial activity against Staphylococcus aureus. Liquid bandage formulations were

prepared using ethanol, ethyl acetate, polyvinyl butyral, polyvinylpyrrolidone, and

polyethylene glycol 400. Galangal essential oil was incorporated at concentrations of 20%,

30%, and 40%. The physical properties including viscosity, water permeation, drying time,

and contact angle were evaluated, along with the antimicrobial efficacy of the galangal

essential oil. The results demonstrated that the formulation containing ethanol : ethyl

acetate : polyvinyl butyral : polyvinylpyrrolidone : polyethylene glycol 400 in the ratio 58 :

30 : 10 : 2 : 0 exhibited a viscosity of 162.8 MPa·s, water permeation of 0.078 g/day, a drying

time of 7.83 minutes, and a contact angle of 53 degrees, closely resembling the physical

characteristics of commercial products. Additionally, inhibition of Staphylococcus aureus

growth was achieved with only 20% galangal essential oil. It can be concluded that the

liquid bandage containing galangal essential oil provides effective wound sealing and shows

antibacterial activity against Staphylococcus aureus.

Keywords: liquid bandage, galangal essential oil, Staphylococcus aureus, film physical

properties, antimicrobial activity