Development of Multilayer Nanofiber Wound Dressings Enhanced with Bio-Selected Stingless Bee and Manuka Honey via Electrospinning

Chotchanit Nakprasit1, Onnalin Sungwiset1, Putcharaporn Wichasan1 and Wirapong Kantamala1

1Princess Chulabhorn Science High School Mukdahan ,281 Moo 6, Ban Nong Hoi, Bang Sai Yai Sub-district, Mueang Mukdahan District, Mukdahan 49000, Thailand

*office@pccm.ac.th

Abstract

Chronic diabetic wounds remain a critical public health challenge, as conventional dressings such as gauze tend to adhere to regenerating tissue, causing re-injury upon removal, and fail to maintain proper moisture balance. This study aimed to develop a multilayer nanofiber wound dressing incorporating the bioactivity of stingless bee honey, a locally sourced bioresource, in comparison with Manuka honey, a widely used medical-grade honey, via electrospinning. Preliminary bioactivity screening using DPPH, protein denaturation, and antiglycation assays showed that stingless bee honey exhibited higher antioxidant activity, while Manuka honey showed superior anti-inflammatory and anti-glycation effects. However, stingless bee honey retained its bioactivity more stably when blended with polyvinyl alcohol (PVA), and was therefore selected for fiber fabrication. The optimal condition, 10% wt honey in PVA at a flow rate of 0.3 mL/hr, produced continuous, bead-free fibers with an average diameter of 456.57 ± 25.64 nm. This active layer was further developed into a bilayer structure by overspinning with polycaprolactone (PCL) and integrating a medical-grade bordered adhesive. The bilayer structure significantly enhanced water resistance, reducing permeability from 25.39% to 0%, increased tensile strength to 5.75 MPa, nearly doubled elongation at break, and sustained over 60% bioactivity for up to 7 days. The prototype caused no damage to simulated skin and withstood more than 50 flexion cycles without detachment. This innovation demonstrates strong potential as an accessible, cost-effective smart wound dressing to advance chronic wound care for diabetic patients.

Keywords: nanofiber wound dressing, stingless bee honey, Manuka honey, electrospinning, diabetic chronic wound.