Assessing Factors Associated with Apparent Hotspot-Cluster Spread in Upper Northern Thailand Using a Rothermel-Inspired Model

Piyachet Saengchan1*, Kritakan Meejan1 and Winwan Santipapwiwatana1

1Princess Chulabhorn Science High School Chiang Rai, 345 Moo 2, Rob Wiang, Mueang Chiang Rai, Chiang Rai 57000, Thailand

*E-mail: Piyachet.barnaby@gmail.com

Abstract

Satellite active-fire data provide broad spatial coverage, but hotspot coordinates represent nominal pixel centres rather than directly observed fire fronts. This study evaluated whether daily centroid displacement of VIIRS hotspot clusters (Robs) could support assessment of fuel moisture, wind speed, and terrain slope in upper northern Thailand. We processed 221,418 detections from Suomi NPP, NOAA-20, and NOAA-21 collected from 1 February to 1 May 2026. After de-duplication and confidence filtering, 186,675 detections remained. Spatial DBSCAN with a 2-km radius, followed by temporal-gap splitting, produced 155 groups with at least two observed days and 1,016 complete Robs records. A validity audit found that a single dominant algorithmic group (Group 0) contained 169,861 detections (92.22% of grouped data), spanned all 90 days, and covered almost the entire 332 × 334 km study box, indicating spatial percolation rather than a single fire. Under physically constrained calibration, the moisture, wind, and slope effects approached zero; the reduced model collapsed to an approximately constant prediction (R2 ≈ 0, RMSE = 2.096 m min⁻¹, MAPE = 225.6%). Row-level correlations with Robs also opposed the expected physical directions. For centroid movements of at least 1 km, the median angular difference from downwind was 85.8° and the Rayleigh p-value was 0.829. These results do not support using daily hotspot-centroid displacement to calibrate a physical fire-spread model. The main contribution is a reproducible validity assessment showing that spatiotemporal event construction and a validated fire-front target must precede factor ranking or model calibration.

Keywords: VIIRS active fire, apparent spread rate, Rothermel model, DBSCAN, validity assessment